| PT NIRWANA BHUMI ENERGY | REGISTER OF HIGH-RISK ACTIVITIES REQUIRING SPECIAL PERMIT | Form No: 0001-FOR-MED-COO-X-2026 Revision: 0 Date: October 2026 |
| --- | --- | --- |
| Standard Operational Reference: 0007-SOP-HSE-COO-X-2026 |  |  |

| Operating Facility / Project Site: | Central Fabrication Yard & Rig Support Base | Assessment Period / Year: | Calendar Year 2026 |
| --- | --- | --- | --- |
| Evaluated By / HSE Officer: | Field Occupational Health Specialist | Approved By / Technical Lead: | Dennis Raditya (Director of Ops & Tech) |
| Surveillance Regulatory Basis: | Permenaker Per.02/MEN/1980 & CSMS | Review Frequency: | Semi-Annual Evaluation |

| No. | Department / Work Unit | High-Risk Operational Activity | Specific Health Hazard Exposure | Required Medical Test Battery |
| --- | --- | --- | --- | --- |
| 1 | Fabrication & Structural Welding | SMAW/GMAW heavy structural welding in enclosed bays | Metal fumes (Mn, Cr, Fe), UV radiation, heat | Spirometry, Chest X-Ray, Visual Acuity, Urine Metals |
| 2 | Abrasive Blasting & Coating | Garnet/silica sandblasting and polyurethane paint spraying | Respirable particulates, isocyanates, organic solvents | Pulmonary Function Test (PFT), Liver Function, Neuro Screen |
| 3 | Rig Equipment Machining & Milling | High-speed lathe cutting and CNC metal grinding | Continuous noise > 88 dBA, metal dust, vibration | Baseline & Semi-Annual Audiometry, Hand-Arm Vibration |
| 4 | Scaffolding & Rigging Services | High-elevation scaffolding erection on client rigs (> 10m) | Height vertigo, physical strain, cardiovascular stress | Vestibular balance, resting ECG, Acrophobia assessment |
| 5 | Chemical Dosing & Tank Cleaning | Cleaning industrial sludge and hydro-testing pipe loops | Chemical toxic vapors, confined space atmosphere, H2S | Biological toxic markers, Serum Cholinesterase, Spirometry |
| 6 | Radiation Non-Destructive Testing | Gamma / X-ray industrial radiography of pipe welds | Ionizing gamma/X-ray radiation | Complete Blood Count (CBC) with differential, TLD badge tracking |

| Prepared By / Company Doctor: | Reviewed By / Technical Authority: | Approved By / Executive Authority: |
| --- | --- | --- |
| Signature / Stamp: | Signature / Stamp: | Signature / Stamp: |
| Name: Occupational Health Specialist (Medical Coordinator) Date: October 2026 | Name: Dennis Raditya (Director of Operations & Tech) Date: October 2026 | Name: Bima Satryo Hartanto (President Director / CEO) Date: October 2026 |